# PT Activity: Configuring basic HSRP (Hot Standby Router Protocol)

## Topology Diagram

## Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| ISP | Gi0/0 | 10.1.1.1 | 255.255.255.252 | N/A |
|  | Gi0/1 | 10.1.1.5 | 255.255.255.252 | N/A |
|  | Gi0/2 | 107.21.3.1 | 255.255.255.0 | N/A |
| BR_R1 | Gi0/0 | 192.168.100.2 | 255.255.255.0 | N/A |
|  | Gi0/1 | 10.1.1.2 | 255.255.255.252 | N/A |
| BR_R2 | Gi0/0 | 192.168.100.3 | 255.255.255.0 | N/A |
|  | Gi0/1 | 10.1.1.6 | 255.255.255.252 | N/A |
| BR_PC | NIC | 192.168.100.100 | 255.255.255.0 | 192.168.100.1 |
| Server | NIC | 107.21.3.223 | 255.255.255.0 | 107.21.3.1 |

## Learning Objectives

Configure STP

Configure OSPF routing and verify functionality

Configure HSRP and verify functionality

Configure Ethernet interface on host PC and test failover

## Introduction

In this activity, you will perform basic STP and OSPF configuration before activating HSRP on the Branch routers.  ISP, BR_R1, BR_R2 have already been preconfigured with hostnames and IP addresses.  The DNS/Web server has also been preconfigured.

Task 1: Configure STP

Configure BR_SW to support Rapid-PVST+ and Port Fast.  This will guarantee quicker failover of HSRP:

BR_SW>enable

BR_SW#config term

BR_SW(config)#spanning-tree mode rapid-pvst

BR_SW(config)#spanning-tree portfast default

Your completion result should be 8%. If not, check for missing configuration statements.

Task 2: Configure OSPF routing

Configure OSPF on ISP, BR_R1 and BR_R2.  Assign all interfaces to Area 0, except for Gi0/2 on ISP.  ISP will advertise a default route to BR_R1 and BR_R2.

Step 1. Enable OSPF on BR_R1

BR_R1(config)#router ospf 1

BR_R1(config-router)#network 192.168.100.0 0.0.0.255 area 0

BR_R1(config-router)#network 10.1.1.0 0.0.0.3 area 0

BR_R1(config-router)#end

Step 2. Enable OSPF on BR_R2

BR_R2(config)#router ospf 1

BR_R2(config-router)#network 192.168.100.0 0.0.0.255 area 0

BR_R2(config-router)#network 10.1.1.4 0.0.0.3 area 0

BR_R2(config-router)#end

Step 3. Enable OSPF on ISP

ISP(config)#router ospf 1

ISP(config-router)#network 10.1.1.0 0.0.0.3 area 0

ISP(config-router)#network 10.1.1.4 0.0.0.3 area 0

ISP(config-router)#default-information originate

ISP(config-router)#exit

ISP(config)#ip route 0.0.0.0 0.0.0.0 Gi0/2

Step 4. Verify routing

Use the show ip route command on BR_R1 and BR_R2 to verify that OSPF is operating correctly.  Both routers should be receiving a default route (type O*E2) from ISP.

BR_R1#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is 10.1.1.1 to network 0.0.0.0

10.0.0.0/8 is variably subnetted, 3 subnets, 2 masks

C       10.1.1.0/30 is directly connected, GigabitEthernet0/1

L       10.1.1.2/32 is directly connected, GigabitEthernet0/1

O       10.1.1.4/30 [110/2] via 192.168.100.3, 00:17:05, GigabitEthernet0/0

[110/2] via 10.1.1.1, 00:17:05, GigabitEthernet0/1

192.168.100.0/24 is variably subnetted, 2 subnets, 2 masks

C       192.168.100.0/24 is directly connected, GigabitEthernet0/0

L       192.168.100.2/32 is directly connected, GigabitEthernet0/0

O*E2 0.0.0.0/0 [110/1] via 10.1.1.1, 00:17:05, GigabitEthernet0/1

Your completion result should be 56%. If not, check for missing configuration statements.

Task 3: Configure HSRP

Configure HSRP group 1 on BR_R1 and BR_R2 using 192.168.100.1 as the standby virtual IP address.  By default, Packet Tracer supports HSRP version 2.  BR_R1 will be configured as the Active HSRP default-gateway and BR_R2 will be configured as Standby.  Preemption is configured on both routers.

Step 1. Enable HSRP on BR_R1

BR_R1(config)#interface gi0/0

BR_R1(config-if)#standby 1 ip 192.168.100.1

BR_R1(config-if)#standby 1 preempt

Step 2. Enable HSRP on BR_R2

BR_R2(config)#interface gi0/0

BR_R2 (config-if)#standby 1 ip 192.168.100.1

BR_R2 (config-if)#standby 1 priority 95

BR_R2 (config-if)#standby 1 preempt

Step 3. Verify HSRP

After a few moments, use the show standby and show standby brief commands on BR_R1 and BR_R2 to verify that HSRP is operating correctly.  BR_R1 should be the Active router and BR_R2 should be Standby.

BR_R1#show standby

GigabitEthernet0/0 - Group 1 (version 2)

State is Active

5 state changes, last state change 00:00:19

Virtual IP address is 192.168.100.1

Active virtual MAC address is 0000.0C9F.F001

Local virtual MAC address is 0000.0C9F.F001 (v2 default)

Hello time 3 sec, hold time 10 sec

Next hello sent in 0.314 secs

Preemption enabled

Active router is local

Standby router is 192.168.100.3

Priority 100 (default 100)

Group name is hsrp-Gig0/0-1 (default)

BR_R1#show standby brief

P indicates configured to preempt.

|

Interface   Grp  Pri P State    Active          Standby         Virtual IP

Gig0/0      1    100 P Active   local           192.168.100.3   192.168.100.1

BR_R2#sh standby brief

P indicates configured to preempt.

|

Interface   Grp  Pri P State    Active          Standby         Virtual IP

Gig0/0      1    95  P Standby  192.168.100.2   local           192.168.100.1

Your completion result should be 82%. If not, check for missing configuration statements.

Task 4: Configure Ethernet interface on host PC and test HSRP failover functionality

Step 1. Configure host PC

- Configure the NIC on BR_PC according to the information in the table.  Also configure BR_PC to use 107.21.3.223 as its DNS server.

Step 2. Test connectivity using Ping

Use a command prompt on BR_PC to Ping the server using the URL http://www.netacad.com

PC>ping www.netacad.com

Pinging 107.21.3.223 with 32 bytes of data:

Reply from 107.21.3.223: bytes=32 time=13ms TTL=126

Reply from 107.21.3.223: bytes=32 time=1ms TTL=126

Reply from 107.21.3.223: bytes=32 time=0ms TTL=126

Reply from 107.21.3.223: bytes=32 time=1ms TTL=126

Ping statistics for 107.21.3.223:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 13ms, Average = 3ms

Step 3. Test connectivity using Tracert

Use a command prompt to trace the physical path taken from BR_PC to the Server.  Confirm that the first hop is the physical address of BR_R1 Gi0/0 interface (192.168.100.2)

PC>tracert www.netacad.com

Tracing route to 107.21.3.223 over a maximum of 30 hops:

1   1 ms      0 ms      1 ms      192.168.100.2

2   1 ms      0 ms      0 ms      10.1.1.5

3   0 ms      1 ms      0 ms      107.21.3.223

Trace complete.

Step 4. Test HSRP failover

From BR_PC, use the ping –t command to start a continuous sequence of pings to the Server.

PC>ping -t www.netacad.com

On BR_R1, shutdown the Gi0/0 interface.

BR_R1#config term

BR_R1(config)#interface gi0/0

BR_R1(config-if)#shutdown

%HSRP-6-STATECHANGE: GigabitEthernet0/0 Grp 1 state Active -> Init

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to administratively down

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to down

00:18:26: %OSPF-5-ADJCHG: Process 1, Nbr 192.168.100.3 on GigabitEthernet0/0 from FULL to DOWN, Neighbor Down: Interface down or detached

Notice that BR_R2 becomes the new Active router.

BR_R2#

%HSRP-6-STATECHANGE: GigabitEthernet0/0 Grp 1 state Standby -> Active

Notice what occurs on BR_PC.  A change in physical gateway has occurred, but this is transparent to the host PC.  It is possible for one or two pings to drop depending on how quickly BR_R2’s hold time expires.  The default hold time for HSRP is 10 seconds.

Reply from 107.21.3.223: bytes=32 time=0ms TTL=126

Reply from 107.21.3.223: bytes=32 time=0ms TTL=126

Reply from 107.21.3.223: bytes=32 time=0ms TTL=126

Request timed out.

Reply from 107.21.3.223: bytes=32 time=1ms TTL=126

Reply from 107.21.3.223: bytes=32 time=1ms TTL=126

Reply from 107.21.3.223: bytes=32 time=1ms TTL=126

Your completion result should be 100%. If not, check for missing configuration statements.

Task 5: Verify HSRP packet exchange

Step 1. Activate BR_R1 Gi0/0 interface to allow the router to reclaim the Active status

BR_R1#config term

BR_R1(config)#interface gi0/0

BR_R1(config-if)#no shutdown

BR_R1(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

%HSRP-6-STATECHANGE: GigabitEthernet0/0 Grp 1 state Standby -> Active

BR_R1(config-if)#

Step 2. Use Simulation mode to view HSRP Hello packets

Enter Simulation mode.  Select only HSRP in the filter window.  Click Auto Capture / Play to see the multicast HSRP Hello packets being sent and received by both BR_R1 and BR_R2.  Confirm the following by observing the PDU details of some of the packets:

- Destination IP address is 224.0.0.102

- UDP port is 1985

- HSRP version is 0x2

- Priority is either 100 or 95 depending on Hello viewed.

- Group number is 1

- Virtual IP is 192.168.100.1

Step 3. Use Simulation mode to view ICMP packet flow from BR_PC to Netacad server

Create a complex PDU.  Use 192.168.100.100 as the source IP address.  Use 107.21.3.223 as the destination IP address.  Use a sequence number if 1 and configure a one-shot time of 5 seconds.  Use the Capture/Forward button to view the ICMP packet flow to and from the Netacad server, via the BR_R1 router.  	Delete the cable between BR_R1 and BR_SW.  Run the simulation again and view the packets flow to and from the server via BR_R2 after it has become the new Active HSRP router.